Internship Report - Day 11

30-Day MERN Stack with AI Internship
TechnoGuide Infosoft Pvt. Ltd., Anand

Day 11 – Emotion Recognition System

On the eleventh day of the internship, I learned about Emotion Recognition Systems using Artificial Intelligence, TensorFlow, and OpenCV. The session focused on real-time facial emotion detection using webcam and mobile camera feeds. I explored how AI models classify human emotions such as Happy, Sad, Angry, Fear, Neutral, Disgust, and Surprised using facial analysis techniques.

The project improved my understanding of Computer Vision, image processing, TensorFlow model integration, OpenCV face detection, and real-time emotion classification systems.

Topics Covered

Introduction to Emotion Recognition

TensorFlow and OpenCV Integration

Face Detection using Haar Cascade

Real-time Webcam Emotion Detection

Android Mobile Camera Integration

Emotion Classification

Transfer Learning

Dataset Preparation

Model Training and Testing

AI-based Facial Analysis

1. Introduction to Emotion Recognition

Emotion Recognition is a Computer Vision application that detects and classifies human emotions using facial expressions.

The system recognizes emotions such as Happy, Sad, Angry, Fear, Neutral, Disgust, and Surprised.

2. Technologies Used

The project uses multiple AI and Computer Vision technologies.

Technologies:
• Python
• TensorFlow
• OpenCV
• NumPy
• Haar Cascade XML

3. Installation Process

Required Python libraries are installed using pip.

Command:
pip install tensorflow opencv-python numpy

Folder Structure:-

4. Face Detection using OpenCV

OpenCV uses Haar Cascade classifiers for detecting human faces from images and video frames.

XML Files Used:
• haarcascade_frontalface_alt.xml
• haarcascade_frontalface_default.xml

5. Webcam Emotion Recognition

The system captures live webcam feed and predicts emotions in real-time.

#Coded by:- Divy Shah

#From:- Technoguide Infosoft

import time

import cv2

import label_image

import os,random

import subprocess

os.environ['TF_CPP_MIN_LOG_LEVEL'] = '2'

size = 4

# We load the xml file

classifier = cv2.CascadeClassifier('haarcascade_frontalface_alt.xml')

global text

webcam = cv2.VideoCapture(0)  # Using default WebCam connected to the PC.

while True:

(rval, im) = webcam.read()

im = cv2.flip(im, 1, 0)  # Flip to act as a mirror

# Resize the image to speed up detection

mini = cv2.resize(im, (int(im.shape[1] / size), int(im.shape[0] / size)))

# detect MultiScale / faces

faces = classifier.detectMultiScale(mini)

# Draw rectangles around each face

for f in faces:

(x, y, w, h) = [v * size for v in f]  # Scale the shapesize backup

sub_face = im[y:y + h, x:x + w]

FaceFileName = "test.jpg"  # Saving the current image from the webcam for testing.

cv2.imwrite(FaceFileName, sub_face)

text = label_image.main(FaceFileName)  # Getting the Result from the label_image file, i.e., Classification Result.

text = text.title()  # Title Case looks Stunning.

font = cv2.FONT_HERSHEY_TRIPLEX

if text == 'Angry':

cv2.rectangle(im, (x, y), (x + w, y + h), (0, 25, 255), 7)

cv2.putText(im, text, (x + h, y), font, 1, (0, 25,255), 2)

if text == 'Smile':

cv2.rectangle(im, (x, y), (x + w, y + h), (0,260,0), 7)

cv2.putText(im, text, (x + h, y), font, 1, (0,260,0), 2)

if text == 'Fear':

cv2.rectangle(im, (x, y), (x + w, y + h), (0, 255, 255), 7)

cv2.putText(im, text, (x + h, y), font, 1, (0, 255, 255), 2)

if text == 'Sad':

cv2.rectangle(im, (x, y), (x + w, y + h), (0,191,255), 7)

cv2.putText(im, text, (x + h, y), font, 1, (0,191,255), 2)

# Show the image/

cv2.imshow('Emotion recognition', im)

key = cv2.waitKey(30)& 0xff

if key == 27:  # The Esc key

break

Run Command:
python recognition_webcam.py

6. Android Mobile Camera Support

The project also supports Android mobile camera feed using the IPWebcam application.

Users must replace the IP address in android_recognition.py with their mobile IP address.

import cv2

import label_image

import os

import  numpy as np

from urllib.request import  urlopen

import time

from playsound import playsound

os.environ['TF_CPP_MIN_LOG_LEVEL'] = '2'

size = 4

# We load the xml file

classifier = cv2.CascadeClassifier('haarcascade_frontalface_alt.xml')

mobile_video="http://192.168.0.101:8080/shot.jpg"

# Using default WebCam connected to the PC.

while True:

img_resp = urlopen(mobile_video)

img_arr = np.array(bytearray(img_resp.read()), dtype=np.uint8)

cap = cv2.imdecode(img_arr, -1)

im=cv2.flip(cap,1,0)

mini = cv2.resize(im, (int(im.shape[1] / size), int(im.shape[0] / size)))

# detect MultiScale / faces

faces = classifier.detectMultiScale(mini)

# Draw rectangles around each face

for f in faces:

(x, y, w, h) = [v * size for v in f]  # Scale the shapesize backup

sub_face = im[y:y + h, x:x + w]

FaceFileName = "test.jpg"  # Saving the current image from the webcam for testing.

cv2.imwrite(FaceFileName, sub_face)

text = label_image.main(FaceFileName)  # Getting the Result from the label_image file, i.e., Classification Result.

text = text.title()  # Title Case looks Stunning.

font = cv2.FONT_HERSHEY_TRIPLEX

if text == 'Angry':

cv2.rectangle(im, (x, y), (x + w, y + h), (0, 25, 255), 7)

cv2.putText(im, text, (x + h, y), font, 1, (0, 25,255), 2)

if text == 'Smile':

cv2.rectangle(im, (x, y), (x + w, y + h), (0,260,0), 7)

cv2.putText(im, text, (x + h, y), font, 1, (0,260,0), 2)

if text == 'Fear':

cv2.rectangle(im, (x, y), (x + w, y + h), (0, 255, 255), 7)

cv2.putText(im, text, (x + h, y), font, 1, (0, 255, 255), 2)

if text == 'Sad':

cv2.rectangle(im, (x, y), (x + w, y + h), (0,191,255), 7)

cv2.putText(im, text, (x + h, y), font, 1, (0,191,255), 2)

# Show the image/

cv2.imshow('Emotion recognition from Android screen', im)

key = cv2.waitKey(30)& 0xff

if key == 27:  # The Esc key

break

7. Dataset Preparation

Training datasets are organized into separate folders for each emotion category.

Emotion Folders:
• Happy
• Sad
• Angry
• Fear
• Neutral
• Disgust
• Surprised

8. Training the AI Model

TensorFlow transfer learning is used for emotion classification training.

Training creates:
• retrained_graph.pb
• retrained_labels.txt

9. Working of the System

The system detects faces, crops the face region, classifies emotions, and displays predictions in real-time.

Process:
Face Detection → Face Cropping → Emotion Classification → Display Result

10. Advantages of the System

Emotion Recognition Systems are useful in modern AI applications.

Advantages:
• Real-time emotion analysis
• AI-powered face recognition
• Smart surveillance support
• Human-computer interaction

11. Real-World Applications

Emotion Recognition is used in various industries and smart AI systems.

Applications:
• Healthcare
• Education
• Security Systems
• AI Assistants
• Customer Analysis

12. Learning Outcome

Basics of Emotion Recognition Systems

TensorFlow and OpenCV Integration

Real-time Face Detection

Webcam and Mobile Camera Handling

Emotion Classification Techniques

Transfer Learning Concepts

AI-based Facial Analysis

Computer Vision Applications

13. Conclusion

The eleventh day of the internship introduced Emotion Recognition Systems using TensorFlow and OpenCV. The practical implementation demonstrated how Artificial Intelligence can analyze human facial expressions and classify emotions in real-time through webcam and mobile camera feeds.

The session enhanced my understanding of Computer Vision, facial analysis, TensorFlow model integration, and AI-powered emotion classification systems. Emotion Recognition technology plays an important role in healthcare, security, education, smart surveillance, and human-computer interaction applications.